EXP 09: ANALYSIS OF PHOTODETECTOR NOISE AND SNR USING MATLAB/SCILAB

OBJECTIVES :

1. Analyze the key noise sources in an optical photodetector, namely shot noise, dark current noise and thermal (receiver) noise.

2. Compute the individual and total noise current variances (σ²) across a range of input optical powers.

3. Evaluate the Signal-to-Noise Ratio (SNR) and observe its scaling in the thermalnoise-limited and shot-noise-limited regimes.

4. Calculate the Minimum Detectable Optical Power (Pmin) at SNR = 1 (0 dB). 5. Simulate and visualize photodetector noise and SNR curves using MATLAB.

SOFTWARE USED : MATLAB

MATLAB PROGRAM:

% Photodetector Noise and SNR Analysis

clc; clear;

close all;

%% 1. Constants and input parameters q = 1.602e-19;  kB = 1.381e-23;  h = 6.626e-34;  c = 2.998e8;

| lambda = 1550e-9; | % wavelength (m) |
| --- | --- |
| eta = 0.85; | % quantum efficiency |
| Id = 5e-9; | % dark current (A) |
| T = 300; | % temperature (K) |
| RL = 50; | % load resistance (ohm) |
| Fn = 2; | % noise figure (3 dB = 2 linear) |
| B = 1e9; | % bandwidth (Hz) |
| Pdbm = -60:0.1:0; | % input power sweep (dBm) |

P      = 10.^((Pdbm-30)/10);   % input power (W)

%% 2. Responsivity and signal current

R  = eta*q*lambda/(h*c);

Ip = R*P;

%% 3. Noise variances and SNR var_shot = 2*q*Ip*B; var_dark = 2*q*Id*B; var_th   = 4*kB*T*B*Fn/RL; var_tot  = var_shot + var_dark + var_th;

SNR_dB   = 10*log10(Ip.^2 ./ var_tot);

%% 4. Minimum detectable power (SNR = 1)

a = R^2;  b = -2*q*B*R;  cc = -(var_dark + var_th);

Pmin    = (-b + sqrt(b^2 - 4*a*cc))/(2*a);

PmindBm = 10*log10(Pmin) + 30;

%% 5. Display results

[~,k] = min(abs(Pdbm+30));          % index of -30 dBm fprintf('Responsivity R      = %.4f A/W\n', R); fprintf('Thermal variance    = %.3e A^2\n', var_th); fprintf('Dark variance       = %.3e A^2\n', var_dark); fprintf('Shot variance(-30)  = %.3e A^2\n', var_shot(k)); fprintf('SNR at -30 dBm      = %.2f dB\n', SNR_dB(k));

fprintf('Pmin (SNR = 1)      = %.2f dBm (%.1f nW)\n', PmindBm, Pmin*1e9);

%% 6.Plots subplot(2,1,1);

loglog(P, var_th*ones(size(P)), P, var_dark*ones(size(P)), P, var_shot, P, var_tot); grid on; xlabel('P_{in} (W)'); ylabel('Noise variance (A^2)'); legend('Thermal','Dark','Shot','Total','Location','best'); subplot(2,1,2);

plot(Pdbm, SNR_dB); hold on; plot([-60 0],[0 0],'r--'); plot([PmindBm PmindBm],[-60

70],'k:'); grid on; xlabel('P_{in} (dBm)'); ylabel('SNR (dB)'); Expected Outputs:

Command Window Output

Responsivity R      = 1.0625 A/W

Thermal variance    = 6.629e-13 A^2

Dark variance       = 1.602e-18 A^2

Shot variance(-30)  = 3.404e-16 A^2

SNR at -30 dBm      = 2.31 dB

Pmin (SNR = 1)      = -31.16 dBm (766.4 nW)

RESULTS:

RUBRICS :

| Objective | Assessment Criterion | Excellent – 4 | Good – 3 | Satisfactory – 2 | Needs Improvement – 1 | Weightage |
| --- | --- | --- | --- | --- | --- | --- |
| Develop the MATLAB program with correct constants and system parameters. | Program Setup and Parameter Definition | Defines all constants and parameters correctly with proper units and a clean script structure, independently. | Defines parameters correctly with minor unit or syntax errors and limited assistance. | Defines most parameters but needs considerable assistance or has unit errors (e.g., dBm/W, linear Fn). | Unable to set up the script or define the parameters. | 25 |
| Compute R, signal current, noise variances and SNR across the power sweep. | Simulation Execution and Data Acquisition | Computes R, Ip, all noise variances and SNR correctly over −60 to 0 dBm and records the values. | Computes all quantities with minor errors or omissions. | Computes some quantities with noticeable errors or inconsistencies. | Unable to run the simulation or record the values. | 25 |
| Obtain the noise and SNR plots and determine Pmin. | Graph PloBing and Pmin Determination | Plots accurate log–log and semi-log curves with labels and reference lines; Pmin within 5% of the reference. | Plots the curves with minor errors; Pmin identified with small deviation. | Plots partial or poorly scaled curves; Pmin approximate. | Unable to plot the curves or determine Pmin. | 25 |
| Interpret noise regimes, SNR slopes and receiver sensitivity. | Analysis and Engineering Interpretation | Correctly identifies thermal- and shot-limited regimes, explains the SNR slopes and links results to receiver design. | Identifies the regimes with minor errors and reasonable interpretation. | Performs basic interpretation with noticeable errors or limited justification. | Unable to interpret the results. | 25 |

REFLECTION :

I learned to analyze shot, dark-current, and thermal noise in a photodetector using MATLAB and calculate reposivity, SNR, and minimum detectable optical power. I observed how noiseaffects receiver sensitivity and verified that SNR increases with input optical power.

RESULT :

The study shows how input optical power affects noise and SNR in a PIN photodetector receiver. Performance depends on the power level:

• Low power: thermal noise dominates, giving negative SNR and no reliable detection (below Pmin ≈ −31.16 dBm).

• Moderate power: the signal rises above the thermal noise and the SNR increases at 20 dB/decade.

• High power: SNR becomes large and shot noise grows linearly, approaching the thermal level near +2.9 dBm.

These results help in estimating the receiver sensitivity and in designing low-noise optical receivers.