SIMATS ENGINEERING

CO2- PROBLEM SOLVING ASSIGNMENT

STANDARDS-COMPLIANT LASER TRANSMITTER DESIGN, WAVELENGTH CONTROL AND NONLINEARITY MANAGEMENT FOR AN 8-CHANNEL DWDM LINK

Course: ECA18 03– Optoelectronics and Optical Communication
Assessment: CO2 Problem-Solving Assignment

Name : KUMARAN.S

Reg no: 192512142

1. PROBLEM REQUIREMENT AND SYSTEM ANALYSIS

1.1 Problem Statement

Dense Wavelength Division Multiplexing (DWDM) is an optical communication technique that transmits multiple independent data channels simultaneously through a single optical fibre using different optical wavelengths. The objective of this assignment is to design and evaluate an appropriate optical transmitter for an eight-channel DWDM link operating around 1550 nm with 100 GHz channel spacing.

Each channel carries 25 Gb/s Non-Return-to-Zero (NRZ) data through six amplified spans, each 80 km long, of standard single-mode fibre. The total transmission distance is therefore:

Total distance = 6 × 80 = 480 km

The proposed transmitter must provide sufficient optical power, high modulation bandwidth, narrow spectral linewidth, stable wavelength operation and reliable transmission while minimizing nonlinear impairments. It must also consider optical safety, applicable interface standards, dispersion compensation and photonic integration.

The principal design challenge is to select an optical source and transmitter configuration that satisfy these requirements while maintaining acceptable signal quality across the complete link.

1.2 System Requirements

| Parameter | Specified value |
| --- | --- |
| Number of DWDM channels | 8 |
| Operating wavelength | Approximately 1550 nm |
| Channel spacing | 100 GHz, approximately 0.8 nm |
| Data rate | 25 Gb/s per channel |
| Modulation format | NRZ |
| Fibre length | 6 × 80 km = 480 km |
| Fibre attenuation | 0.2 dB/km |
| Nonlinear coefficient | 1.3 W⁻¹km⁻¹ |
| Effective fibre area | 80 µm² |
| Launch power | 2 mW per channel |
| Laser threshold current | 12 mA |
| Bias current | 40 mA |
| Slope efficiency | 0.3 mW/mA |
| Modulation bandwidth | 20 GHz |
| Ambient temperature variation | 40 °C |
| Required wavelength tolerance | ±0.1 nm |

1.3 Problem Requirement Statement

The proposed transmitter must generate eight wavelength-separated optical channels with 25 Gb/s NRZ data per channel and approximately 200 Gb/s aggregate data capacity. It must maintain the assigned wavelengths within the required tolerance, provide adequate optical power and modulation bandwidth, and control dispersion, nonlinear effects and crosstalk. The design must incorporate appropriate wavelength stabilization, optical safety measures, applicable standards verification and a practical photonic integration strategy.

2. OPTICAL SOURCE SELECTION

2.1 Emission Wavelength and Quantum Efficiency

The given bandgap energy is 0.80 eV. The approximate emission wavelength is calculated using:

λ (µm) = 1.24 / E_g (eV)

Substituting the given value:

λ = 1.24 / 0.80 = 1.55 µm = 1550 nm

Therefore, the calculated emission wavelength matches the intended operating window.

The internal quantum efficiency is calculated from the radiative and non-radiative carrier lifetimes:

ηᵢ = τ_nr / (τ_r + τ_nr)

Given:

τ_r = 10 ns; τ_nr = 40 ns

ηᵢ = 40 / (10 + 40) = 40 / 50 = 0.80

Internal quantum efficiency = 80%.

This result indicates that, under the simplified lifetime model, 80% of the recombination events are radiative. Actual external efficiency also depends on optical confinement, extraction losses and device design.

2.2 Comparison of Optical Sources

| Optical source | Characteristics | Suitability for this link |
| --- | --- | --- |
| LED | Broad spectrum, low optical power and limited high-speed performance | Unsuitable for long-distance DWDM |
| Fabry–Perot laser | Multiple longitudinal modes and comparatively broad spectrum | Less suitable because of spectral overlap and dispersion concerns |
| DBR laser | Distributed Bragg reflector provides wavelength-selective feedback | Suitable for narrow-linewidth, wavelength-controlled applications |
| DFB laser | Distributed feedback supports stable, single-longitudinal-mode operation | Highly suitable for high-speed DWDM |
| VCSEL | Compact, efficient and suitable for many short-reach applications | Possible alternative for suitable wavelength and fibre-interface designs |
| Quantum-dot laser | Potential advantages in temperature stability and threshold behaviour, depending on device design | Promising alternative if the required wavelength and performance are demonstrated |

The source must support a stable single-mode spectrum, sufficient modulation speed and appropriate fibre coupling. A narrow linewidth and high side-mode suppression ratio (SMSR) reduce spectral spreading and unwanted channel interference. However, actual linewidth and SMSR values must be obtained from the selected device datasheet.

2.3 Selected Optical Source

Design Decision 1: I recommend a temperature-controlled DFB laser source for each DWDM wavelength channel.

The selection is justified by the following considerations:

- Its single-mode operation is suitable for wavelength-separated DWDM transmission.

- It supports high-speed modulation when the selected device has adequate bandwidth.

- Its narrow spectral linewidth can reduce phase-noise and stimulated Brillouin scattering concerns.

- Its wavelength can be stabilized using temperature control and appropriate drive-current regulation.

- It is suitable for integration into multi-channel optical transmitter assemblies.

A DBR laser is a technically credible alternative when wider wavelength tuning or particular wavelength-selection requirements make it preferable. The final device must be verified against the required power, linewidth, SMSR and temperature specifications.

3. MODULATION, OUTPUT POWER AND WAVELENGTH CONTROL

3.1 Optical Output Power

The output power at the specified bias current is calculated using:

P = η_s (I − I_th)

Given:

η_s = 0.3 mW/mA, I = 40 mA and I_th = 12 mA

P = 0.3 × (40 − 12)

P = 0.3 × 28 = 8.4 mW

Conversion into dBm:

P(dBm) = 10 log₁₀[P(mW)]

P(dBm) = 10 log₁₀(8.4) = 9.24 dBm

This is the calculated laser output power using the supplied linear slope-efficiency model. It is not automatically the power launched into the fibre because coupling losses, modulation losses, connectors and attenuation may reduce the available power.

The specified launch power is 2 mW per channel, equivalent to:

P_launch(dBm) = 10 log₁₀(2) = 3.01 dBm per channel

Thus, the laser output estimate exceeds the specified launch level, leaving a nominal power difference of 6.23 dB before accounting for actual component losses. An optical attenuator or suitable power-control stage may be used where necessary.

3.2 Modulation Bandwidth

The approximate bandwidth requirement for NRZ data is:

B ≈ 0.7R_b

Given R_b = 25 Gb/s:

B ≈ 0.7 × 25 = 17.5 GHz

The specified modulation bandwidth is 20 GHz.

Bandwidth margin = 20 − 17.5 = 2.5 GHz

Percentage margin = (2.5 / 17.5) × 100 = 14.29%

The candidate bandwidth is adequate according to the supplied approximation, with a 2.5 GHz margin. Nevertheless, practical performance must also be verified through the complete electro-optical response, rise time, chirp and eye-diagram measurements.

3.3 Carrier and Photon Rate Equations

A simplified semiconductor laser model uses the carrier rate equation:

dN/dt = I/(qV) − R(N) − Γv_g g(N,S)S

The photon rate equation is:

dS/dt = Γv_g g(N,S)S + β_sp R_sp(N) − S/τ_p

Here, N represents carrier density, S represents photon density, I is injection current, q is electronic charge, V is active-region volume, R(N) is the carrier recombination rate, Γ is the optical confinement factor, v_g is group velocity, g is optical gain, β_sp is the spontaneous-emission coupling factor and τ_p is photon lifetime.

Relaxation oscillations arise from the coupled dynamics of carrier density and photon density. Small-signal modulation describes operation around a steady bias point, whereas large-signal modulation involves larger excursions and may produce nonlinear transient behaviour, pattern dependence and overshoot. The exact relaxation-oscillation frequency cannot be calculated from the supplied information because the necessary differential gain, photon lifetime and operating-point parameters are not specified.

3.4 Direct Versus External Modulation

Direct modulation changes the laser injection current to encode data. It is comparatively simple and economical but can produce frequency chirp and interact with chromatic dispersion.

External modulation uses a separate device, such as a Mach–Zehnder modulator, to encode the data while the laser operates at a more stable bias point. It offers better control of chirp and is appropriate when transmission distance and spectral stability are important.

3.5 Temperature Stabilization

The uncontrolled wavelength drift is:

Δλ = wavelength drift per °C × temperature variation

Δλ = 0.1 × 40 = 4 nm

The drift is five times the approximate 0.8 nm channel spacing. It is also 40 times the permitted absolute wavelength deviation of 0.1 nm.

The maximum temperature deviation corresponding to ±0.1 nm is:

ΔT = 0.1 nm / (0.1 nm/°C) = ±1 °C

A thermoelectric cooler (TEC) with a temperature sensor and feedback controller should therefore stabilize the laser temperature to approximately ±1 °C under the given linear drift model. Additional margin may be needed for sensor error, package gradients and other wavelength-shift mechanisms.

Injection-current control can provide fine wavelength tuning, but it also changes carrier density and may affect output power and chirp. TEC control is recommended as the primary stabilization mechanism, with current control used for bias regulation and carefully calibrated fine tuning.

Design Decision 2: I recommend external modulation with TEC-based wavelength stabilization because it offers better control of chirp and wavelength drift for the 480 km DWDM link.

4. NONLINEAR IMPAIRMENTS AND THEIR CONTROL

4.1 Effective Fibre Length

The attenuation coefficient in the effective-length equation must be expressed in inverse kilometres, not dB/km.

Given attenuation = 0.2 dB/km:

α = 0.2 / 4.343 = 0.04605 km⁻¹

For an 80 km span:

L_eff = [1 − exp(−αL)] / α

L_eff = [1 − exp(−0.04605 × 80)] / 0.04605

L_eff ≈ 21.72 km

4.2 Self-Phase Modulation

Self-phase modulation (SPM) occurs when the intensity-dependent refractive index of the fibre causes the signal to modify its own optical phase.

The nonlinear phase shift is:

φ_NL = γPL_eff

Given γ = 1.3 W⁻¹km⁻¹ and P = 2 mW = 0.002 W:

φ_NL = 1.3 × 0.002 × 21.72

φ_NL ≈ 0.0565 rad per span

Assuming the same effective launch power is restored at each span and the phase shifts accumulate:

φ_NL,total ≈ 6 × 0.0565 = 0.339 rad

This is an idealized estimate. Actual accumulation depends on amplifier placement, signal-power evolution, dispersion and interactions among channels.

4.3 Cross-Phase Modulation and Four-Wave Mixing

Cross-phase modulation (XPM) occurs when the intensity of one channel changes the phase of another channel through the Kerr effect. Its impact depends on channel power, pulse overlap, walk-off and fibre dispersion.

Four-wave mixing (FWM) involves interactions among optical frequencies that generate additional frequency components. These components can interfere with useful DWDM channels. The 100 GHz channel spacing establishes the frequency relationships, while phase matching and dispersion influence FWM efficiency.

To control these impairments, the design should use controlled launch power, suitable channel spacing, dispersion-aware system planning and appropriate optical filtering. The actual severity of XPM and FWM cannot be quantified from channel spacing alone.

4.4 SBS and SRS Thresholds

The supplied threshold equations are:

P_th(SBS) ≈ 21A_eff/(g_B L_eff)

P_th(SRS) ≈ 16A_eff/(g_R L_eff)

For consistent SI units:

A_eff = 80 × 10⁻¹² m²

L_eff = 21.72 km = 21,720 m

g_B = 5 × 10⁻¹¹ m/W

g_R = 6 × 10⁻¹⁴ m/W

Stimulated Brillouin scattering (SBS):

P_th(SBS) = [21 × 80 × 10⁻¹²] / [5 × 10⁻¹¹ × 21,720]

P_th(SBS) ≈ 0.00155 W = 1.55 mW

Stimulated Raman scattering (SRS):

P_th(SRS) = [16 × 80 × 10⁻¹²] / [6 × 10⁻¹⁴ × 21,720]

P_th(SRS) ≈ 0.983 W = 983 mW

Comparison with the specified launch power:

- Per-channel power: 2 mW.

- Total eight-channel power: 8 × 2 = 16 mW.

- Estimated SBS threshold: 1.55 mW.

- Estimated SRS threshold: 983 mW.

The 2 mW per-channel power exceeds the calculated SBS threshold, indicating that SBS requires attention under the simplified threshold model. The 16 mW total power is below the calculated SRS threshold. However, SBS and SRS thresholds are approximate and depend on linewidth, polarization, fibre characteristics and signal conditions. Multichannel power must not be treated as a single-frequency SBS input without accounting for the individual channel spectra.

SBS results from interaction between light and acoustic waves, generally producing backward-scattered light. SRS transfers optical power through interaction with vibrational modes in the fibre and can transfer energy toward lower optical frequencies. A broader source linewidth can increase the effective SBS threshold, provided the broadening is compatible with the DWDM spectral requirements.

4.5 Eye Safety and Protective Earthing

The total launched optical power is 16 mW, equivalent to approximately 12.04 dBm. This value alone does not establish an ANSI Z136 laser safety classification. The assessment must consider accessible emission, wavelength, beam characteristics, exposure duration, enclosure design and the applicable standard.

For preliminary screening only, if a project adopts an illustrative 10 mW accessible-output limit, 16 mW would exceed that assumed limit. This is not a universal ANSI Z136 limit and must not be represented as proof of compliance or non-compliance. The actual accessible emission and laser classification must be determined before deployment.

Practical safety measures include enclosed optical paths, connector interlocks where appropriate, laser warning labels, power monitoring and safe fibre inspection procedures. Optical connectors must never be viewed directly when a laser may be active.

Protective earthing: The transmitter chassis, metal enclosure and exposed conductive parts of mains-powered equipment must be connected to protective earth according to the equipment manufacturer's instructions and applicable electrical codes. Protective earth must not be replaced by signal ground, an optical-fibre shield or an improvised connection. Fibre itself does not provide a protective-earth path. Earth-electrode dimensions and earth resistance cannot be specified from this assignment because the installation supply, soil conditions and applicable electrical requirements are not provided.

Design Decision 3: I recommend a controlled-launch-power strategy, dispersion-aware channel planning, suitable filtering and formal optical safety verification. The calculated SBS threshold indicates that launch power and linewidth require particular attention. Any mitigation must preserve adequate received signal quality.

5. SOLITON FEASIBILITY, DIGITAL BACKPROPAGATION AND INTEGRATION

5.1 Dispersion Length

Soliton transmission requires a balance between anomalous chromatic dispersion and Kerr nonlinearity.

The dispersion length is:

L_D = T₀² / |β₂|

Given T₀ = 15 ps and |β₂| = 20 ps²/km:

L_D = 15² / 20 = 225 / 20

L_D = 11.25 km

5.2 Fundamental Soliton Peak Power

For a fundamental soliton, L_NL = L_D.

Since L_NL = 1/(γP₀):

P₀ = 1/(γL_D)

P₀ = 1/(1.3 × 11.25)

P₀ = 0.06838 W = 68.38 mW

This is the calculated peak power required for a fundamental soliton under the supplied model and assumed anomalous-dispersion conditions.

5.3 Soliton Order at the Specified Launch Power

At P₀ = 2 mW:

L_NL = 1/(γP₀)

L_NL = 1/(1.3 × 0.002)

L_NL = 384.62 km

The soliton order is:

N² = L_D/L_NL

N = √(11.25/384.62)

N ≈ 0.171

Since N is substantially below unity, the specified launch power does not satisfy the fundamental-soliton condition. Therefore, soliton transmission is not recommended under the given operating conditions. The calculated peak-power requirement also assumes that the stated pulse width is the soliton parameter T₀ and that the required dispersion regime is available.

5.4 Digital Backpropagation

Digital backpropagation (DBP) numerically reverses propagation effects by modelling chromatic dispersion and Kerr nonlinearity. It can improve signal quality when accurate channel information and suitable receiver processing are available.

Its limitations include computational complexity, sensitivity to model mismatch and the need for suitable sampled optical-field information. Full multi-channel compensation is especially demanding because interactions among all eight channels must be represented.

For the stated 25 Gb/s NRZ link, full-link DBP should not be assumed to be directly available without specifying a compatible coherent receiver and DSP architecture. The recommended baseline is controlled launch power, dispersion-aware link design and receiver equalization appropriate to the selected detection format. DBP may be evaluated as an optional enhancement if the receiver architecture supports it.

5.5 Standards Verification

ITU-T G.957 specifies optical interface characteristics for relevant synchronous digital hierarchy systems. IEEE 802.3bs and IEEE 802.3bm define Ethernet physical-layer implementations, including high-speed optical interfaces.

These references must be checked against the actual system architecture and applicable clauses. They do not automatically establish compliance for an arbitrary eight-channel DWDM transmitter. Verification should include the relevant wavelength range, transmitter power, spectral characteristics, modulation requirements, receiver compatibility and interface limits.

ANSI Z136 requirements must be addressed separately for laser safety.

5.6 Photonic Integration

| Implementation | Advantages | Limitations |
| --- | --- | --- |
| Discrete components | Flexible selection, accessible testing and replacement | Larger footprint, more interconnections and coupling interfaces |
| Monolithic InP integration | Compact optical integration and reduced interconnect count | Process complexity, thermal design and fabrication cost |
| Si-photonics with hybrid laser | Potential scalability, compact routing and integration with electronic control | Requires hybrid laser integration and careful coupling and thermal management |

For an eight-channel transmitter, silicon photonics with hybrid-integrated DFB lasers is recommended as the development target because it can support compact multi-channel routing and scalable integration. This recommendation is conditional on fabrication access, thermal performance, channel isolation, reliability and demonstrated optical performance. A discrete prototype is appropriate for initial laboratory validation.

Design Decision 4: I recommend a silicon-photonics transmitter with hybrid-integrated, temperature-controlled DFB lasers, external modulators and appropriate receiver-side compensation. This approach offers a scalable implementation while preserving the spectral and modulation control needed for DWDM operation. Full multi-channel DBP should remain optional until its receiver compatibility and processing requirements are established.

6. FINAL TRANSMITTER DESIGN AND VERIFICATION

6.1 Proposed Transmitter Block Diagram

8DFB LASER CHANNELS

1550nm REGION/ 100 GHZ GRID

TEC + TEMPERATURE FEEDBACK

NRZ DRIVER CIRCUITS

DATA INPUTS

CH1,CH2,…CH8

8- CHANNEL DWDM MUX

OPTICAL POWER CONTROL AND CHANNEL MONITORING

EXTERNAL MOUDLATORS (MACH-ZEHDER)

DWDM DEMUX

SINGLE – MODE FIBRE LINK 6 x 80 km SPANS

WITH OPTICAL AMPLIFIERS

OPTICAL OUTPUT

2mW PER CHANNEL

RECEIVER CHANNELS + APPROPRIATE DSP/EQ

The diagram represents the proposed architecture. The exact amplifier arrangement, multiplexer insertion loss, receiver design and compensation implementation must be established during detailed system design.

6.2 Final Parameter Table

| Design parameter | Proposed value or decision |
| --- | --- |
| Optical source | DFB laser per channel |
| Calculated emission wavelength | 1550 nm |
| Number of channels | 8 |
| Channel spacing | 100 GHz |
| Per-channel data rate | 25 Gb/s NRZ |
| Aggregate data rate | 200 Gb/s |
| Calculated laser output | 8.4 mW or 9.24 dBm |
| Specified fibre launch power | 2 mW or 3.01 dBm per channel |
| Modulation bandwidth | 20 GHz |
| Required approximate bandwidth | 17.5 GHz |
| Bandwidth margin | 2.5 GHz |
| Wavelength control | TEC with feedback |
| Temperature tolerance from drift model | Approximately ±1 °C |
| Modulation configuration | External Mach–Zehnder modulation |
| Effective fibre length | 21.72 km per span |
| SPM phase shift | 0.0565 rad per span |
| Six-span SPM estimate | 0.339 rad |
| SBS threshold estimate | 1.55 mW |
| SRS threshold estimate | 983 mW |
| Dispersion length | 11.25 km |
| Fundamental-soliton peak power | 68.38 mW |
| Soliton order at 2 mW | 0.171 |
| Soliton transmission | Not recommended at the stated power |
| Integration target | Silicon photonics with hybrid lasers |
| Standards and safety | Verify applicable interface clauses and ANSI Z136 requirements |

6.3 Final Verification Table

| Requirement | Result | Assessment |
| --- | --- | --- |
| Optical source | DFB laser selected | Suitable, subject to datasheet verification |
| Output power | 8.4 mW calculated | Nominally adequate; coupling losses must be included |
| Modulation bandwidth | 20 GHz versus 17.5 GHz required | Satisfies the given approximate criterion |
| Wavelength stability | Uncontrolled drift of 4 nm | TEC stabilization required |
| Nonlinear effects | SBS threshold below the specified per-channel launch power | Further validation and mitigation required |
| SRS | Calculated threshold much greater than launch power | Below the simplified estimated threshold |
| Soliton operation | N ≈ 0.171 | Fundamental-soliton condition not met |
| Compensation | Dispersion-aware design and appropriate receiver equalization | Feasible subject to receiver specification |
| Integration | Silicon photonics with hybrid lasers | Promising, subject to implementation validation |
| Standards compliance | Exact interface applicability must be checked | Not yet demonstrated |
| Laser safety | Accessible emission and classification must be assessed | Not yet demonstrated |
| Protective earthing | Follow equipment instructions and applicable electrical codes | Installation verification required |

7. CONCLUSION AND FINAL ENGINEERING RECOMMENDATION

The analysis supports the selection of temperature-controlled DFB lasers operating around 1550 nm for the eight-channel DWDM transmitter. The calculated emission wavelength is 1550 nm, and the internal quantum efficiency is 80%. At the specified bias current, the simplified output-power model gives 8.4 mW, while the required NRZ bandwidth is approximately 17.5 GHz. The candidate 20 GHz bandwidth provides a 2.5 GHz margin.

Temperature stabilization is essential because the predicted uncontrolled wavelength drift is 4 nm, which exceeds the allowed ±0.1 nm tolerance. External modulation and TEC feedback are therefore recommended to reduce chirp-related penalties and stabilize the wavelength grid.

The nonlinear analysis gives an effective length of approximately 21.72 km per span and an idealized six-span SPM phase accumulation of approximately 0.339 rad. The calculated SBS threshold of approximately 1.55 mW is lower than the specified 2 mW per-channel launch power, indicating that SBS must be investigated and controlled. The calculated SRS threshold is approximately 983 mW. These estimates require confirmation using the selected fibre, actual channel linewidth and amplifier power profile.

The soliton analysis produces a dispersion length of 11.25 km and a soliton order of approximately 0.171 at the specified launch power. Consequently, soliton transmission is not recommended under the given conditions. Dispersion-aware engineering and receiver-side compensation are more appropriate for the proposed baseline system.

Silicon-photonics integration with hybrid DFB lasers and external modulators is recommended as the target implementation, provided that its optical performance, thermal behaviour and reliability are demonstrated. The design must also undergo formal optical safety assessment, protective-earthing verification and applicable standards testing before installation.

Final deployment decision: NO — not for immediate deployment. The proposed architecture is technically justified as a design candidate, but deployment should proceed only after verification of the exact DWDM wavelength plan, amplifier and receiver budgets, SBS mitigation, temperature stability, applicable standards, accessible laser emission and electrical protective earthing. Once these checks have been completed successfully, the transmitter may be considered for deployment.