SIMATS ENGINEERING

CO 5 -  CASE STUDY BASED ASSIGNMENT

TITLE: DWDM Metro Link Commissioning: From Field Measurements to a Standards-Based Upgrade Decision

ECA1803 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

NAME : KUMARAN.S

REG NO: 192512142

STAGE 1 – INTERPRET THE MEASUREMENT AND PERFORMANCE REQUIREMENT

1.1 Introduction

Dense Wavelength Division Multiplexing (DWDM) enables multiple optical channels to transmit simultaneously through a single optical fibre. The given metro network consists of a 60 km ITU-T G.652.D single-mode fibre link carrying eight 10 Gb/s NRZ channels on a 100 GHz DWDM grid. The aggregate transmission capacity is:

Aggregate capacity = Number of channels × Data rate per channel

Aggregate capacity = 8 × 10 Gb/s = 80 Gb/s.

The operator intends to upgrade the link to 1.6 Tb/s without installing new fibre. However, Channel 5 exhibits a partially closed eye and a high bit error rate, while Channel 3 has an apparently open eye but insufficient receiver margin. Therefore, the existing link requires fault localization, link-budget correction, channel-specific diagnosis, and standards-based upgrade planning.

1.2 Measurement Instruments

The optical power meter measures the received optical power and helps verify the link budget. The optical spectrum analyser (OSA) measures wavelength spacing, channel power, spectral occupancy, and optical signal-to-noise ratio. The optical time-domain reflectometer (OTDR) identifies fibre events, splice losses, connector reflections, and approximate event locations.

The eye-diagram and BER tester evaluates signal quality, jitter, eye opening, and bit error performance. A dispersion or PMD analyser measures chromatic dispersion and polarization mode dispersion.

The principal standards are ITU-T G.652.D for the fibre characteristics and ITU-T G.694.1 for DWDM frequency-grid planning.

1.3 Performance Requirement Statement

The repaired link must support the intended 1.6 Tb/s upgrade while maintaining adequate receiver power, a minimum 3 dB system margin, acceptable OSNR, and BER no greater than 10^-12. Channel 5 must achieve an acceptable extinction ratio and eye opening. All excessive-loss connectors and splices must be repaired, and the selected upgrade must satisfy the applicable frequency-grid, optical-interface, spectral-efficiency, and energy requirements.

- STAGE 2 – LINK-BUDGET, DISPERSION AND OSNR ANALYSIS

2.1 Fibre Attenuation

Given:

Fibre length = 60 km

Fibre attenuation = 0.21 dB/km

Fibre attenuation loss = Fibre length × Attenuation per kilometre

Fibre attenuation loss = 60 × 0.21

Fibre attenuation loss = 12.60 dB.

2.2 Design Link Loss

The design allows four connectors at 0.5 dB each and 13 splices at 0.05 dB each.

Connector loss = 4 × 0.5 = 2.00 dB.

Splice loss = 13 × 0.05 = 0.65 dB.

Total design loss = Fibre loss + Connector loss + Splice loss

Total design loss = 12.60 + 2.00 + 0.65

Total design loss = 15.25 dB.

2.3 As-Built Link Loss

The measured data indicate three connectors at 0.5 dB each and 12 fusion splices at 0.05 dB each. Event 2 is the thirteenth, mechanical splice.

Measured connector loss = 3 × 0.5 = 1.50 dB.

Measured fusion-splice loss = 12 × 0.05 = 0.60 dB.

Mechanical-splice loss = 0.62 dB.

Event 3 loss = 1.20 dB.

Event 4 has an unresolved loss, so its loss must not be added as a known numerical value.

Known as-built loss = Fibre loss + Measured connector loss + Fusion-splice loss + Mechanical-splice loss + Event 3 loss

Known as-built loss = 12.60 + 1.50 + 0.60 + 0.62 + 1.20

Known as-built loss = 16.52 dB.

The known as-built loss is therefore 16.52 dB, excluding any unresolved loss at Event 4 and other losses not specified in the case.

The design allowances must be reconciled with the actual component inventory before a final field loss is declared. In particular, the stated design allowance of four connectors differs from the three connectors explicitly included in the measured-loss list.

2.4 Received Optical Power

Given:

Transmitter line output power per channel = +1 dBm.

Known as-built loss = 16.52 dB.

Demultiplexer loss = 6.0 dB.

Received power before additional penalties = Transmitter power − Link loss − Demultiplexer loss

Received power = +1 − 16.52 − 6.0

Received power = -21.52 dBm.

The receiver sensitivity is -24 dBm.

Available sensitivity margin = Received power − Receiver sensitivity

Available sensitivity margin = -21.52 − (-24)

Available sensitivity margin = 2.48 dB.

The required design margin is 3 dB. Therefore, the known-loss budget falls short by:

Margin shortfall = 3.00 − 2.48 = 0.52 dB.

This calculation excludes any additional dispersion and extinction-ratio penalties.

2.5 Dispersion Penalty

Given:

Dispersion parameter, D = 17 ps/(nm·km).

Fibre length, L = 60 km.

Source RMS spectral width, sigma_lambda = 0.03 nm.

Bit rate, B = 10 Gb/s = 10 × 10^9 bits/s.

The supplied relation is:

Dispersion penalty = 5 log10[1 + 2 pi (B × D × L × sigma_lambda)^2].

The expression requires consistent units. Convert the dispersion parameter to seconds per metre:

D = 17 ps/(nm·km) = 17 × 10^-6 s/m^2.

Convert spectral width:

sigma_lambda = 0.03 nm = 3 × 10^-11 m.

Calculate the dimensionless term:

X = B × D × L × sigma_lambda

X = (10 × 10^9) × (17 × 10^-6) × 60 × (3 × 10^-11)

X = 3.06 × 10^-4.

Dispersion penalty = 5 log10[1 + 2 pi X^2]

Dispersion penalty = approximately 0.00000128 dB.

This result is negligible under the supplied formula and assumptions. The formula's applicability and unit convention should be checked against the course reference before treating this value as a validated physical estimate.

2.6 Extinction-Ratio Penalty

The case provides the extinction ratios of Channel 3 and Channel 5 but does not provide an explicit formula converting extinction ratio into receiver-power penalty. Therefore, an exact penalty should not be invented.

Channel 3 extinction ratio = 9.0 dB.

Channel 5 extinction ratio = 4.5 dB.

Required extinction ratio = 8.2 dB.

Channel 3 meets the extinction-ratio requirement because 9.0 dB is greater than 8.2 dB.

Channel 5 fails the extinction-ratio requirement because 4.5 dB is less than 8.2 dB.

The Channel 5 transmitter or optical modulation path must therefore be investigated and corrected before its receiver margin can be accepted.

2.7 Optical Signal-to-Noise Ratio

The supplied relation is:

OSNR = 58 + P_in − NF.

Given:

Amplifier input power per channel = -6 dBm.

Amplifier noise figure = 5 dB.

OSNR = 58 + (-6) − 5

OSNR = 47 dB.

The calculated value is 47 dB under the supplied relation. However, this expression represents an assumed calculation model and does not by itself establish the OSNR at the receiver after fibre propagation, demultiplexer losses, and other noise contributions.

The measured receiver OSNR must be verified using the OSA with a consistent reference bandwidth of 0.1 nm. The required OSNR is 20 dB for the legacy system.

2.8 Polarization Mode Dispersion

Given:

PMD coefficient = 0.1 ps/sqrt(km).

Fibre length = 60 km.

PMD delay = PMD coefficient × sqrt(Fibre length)

PMD delay = 0.1 × sqrt(60)

PMD delay = 0.775 ps approximately.

The bit period at 10 Gb/s is:

Bit period = 1 / Bit rate

Bit period = 1 / (10 × 10^9)

Bit period = 100 ps.

Ten percent of the bit period = 0.1 × 100 = 10 ps.

Since 0.775 ps is less than 10 ps, the estimated PMD is within the specified 10% tolerance.

DESIGN DECISION 1 – LINK-BUDGET CONFIGURATION

I recommend repairing the excessive-loss events and correcting Channel 5 before changing the launch power. The known as-built loss is 16.52 dB, and the received power is -21.52 dBm after the stated demultiplexer loss. This gives a 2.48 dB margin, which is 0.52 dB below the required 3 dB margin even before additional penalties. The 1.20 dB connector event is the dominant known excess-loss event and should be repaired first.

- STAGE 3 – OTDR FAULT LOCALIZATION, SPLICE AND CONNECTOR CONFIGURATION

3.1 OTDR Pulse Width Selection

The OTDR pulse-width options are:

10 ns: Dynamic range = 20 dB, event dead zone = 1 m, attenuation dead zone = 4 m.

100 ns: Dynamic range = 27 dB, event dead zone = 5 m, attenuation dead zone = 15 m.

1 microsecond: Dynamic range = 33 dB, event dead zone = 40 m, attenuation dead zone = 100 m.

The approximate spatial pulse length is:

Spatial pulse length = c × Pulse duration / (2 × Group index).

Given:

c = 2.998 × 10^8 m/s.

Group index = 1.4682.

For 10 ns:

Spatial pulse length = (2.998 × 10^8 × 10 × 10^-9) / (2 × 1.4682)

Spatial pulse length = approximately 1.02 m.

For 100 ns:

Spatial pulse length = (2.998 × 10^8 × 100 × 10^-9) / (2 × 1.4682)

Spatial pulse length = approximately 10.21 m.

For 1 microsecond:

Spatial pulse length = (2.998 × 10^8 × 1 × 10^-6) / (2 × 1.4682)

Spatial pulse length = approximately 102.10 m.

Events 3 and 4 are separated by only 9 m. A 10 ns pulse is the best available setting for resolving them because it has the smallest spatial pulse length and the smallest event dead zone. However, the stated 1 m event dead zone does not guarantee that both losses can be measured accurately, particularly when the first event is highly reflective. The 100 ns and 1 microsecond pulses are unsuitable for resolving a 9 m separation reliably.

3.2 OTDR Dynamic-Range Requirement

The minimum dynamic range requirement is:

Required dynamic range = One-way trace loss + 6 dB.

Using the known as-built loss:

Required dynamic range = 16.52 + 6

Required dynamic range = 22.52 dB.

The 10 ns setting has 20 dB dynamic range and does not satisfy the requirement.

The 100 ns setting has 27 dB dynamic range and satisfies the numerical requirement.

The 1 microsecond setting has 33 dB dynamic range and also satisfies the numerical requirement, but its large event and attenuation dead zones prevent reliable separation of the two connectors only 9 m apart.

Therefore, no single listed pulse width satisfies both the calculated dynamic-range criterion and the requirement to resolve Events 3 and 4. Use the 10 ns pulse for local event separation and a 100 ns pulse for the longer-range trace. If the equipment permits, use a suitable launch fibre and receive fibre and test from both ends to improve event characterization. A higher-dynamic-range OTDR or an alternate test configuration may be required for a fully valid combined measurement.

3.3 Interpretation of OTDR Events

Event 1 occurs at 12.00 km and has a loss of 0.05 dB with no reflection. This is consistent with a low-loss fusion splice. It meets the stated 0.05 dB splice allowance.

Event 2 occurs at 31.50 km and has a loss of 0.62 dB with reflectance of -34 dB. This is consistent with a high-loss mechanical splice or a poor mechanical joint. It exceeds the 0.05 dB splice allowance by:

Excess loss = 0.62 − 0.05 = 0.57 dB.

The recommended action is to replace the mechanical splice with a properly prepared fusion splice, provided the fibre condition and repair procedure permit it.

Event 3 occurs at 59.90 km and has a loss of 1.20 dB with reflectance of -22 dB. The event is identified as an LC/UPC patch-panel connector. Its loss and reflection indicate a poor connection, contamination, damage, or an improperly seated connector. It is the highest-priority known-loss event.

Event 4 occurs at 59.909 km and has reflectance of -40 dB, but its loss is unresolved. It is the second connector of a patch cord, only 9 m from Event 3. The available trace does not permit an accurate separate loss estimate for this event.

3.4 Event Location and Round-Trip Delay

The OTDR round-trip time is:

t = 2 × n × L / c.

For Event 1 at 12.00 km:

t = (2 × 1.4682 × 12,000) / (2.998 × 10^8)

t = approximately 117.53 microseconds.

For Event 2 at 31.50 km:

t = (2 × 1.4682 × 31,500) / (2.998 × 10^8)

t = approximately 308.66 microseconds.

For Event 3 at 59.90 km:

t = (2 × 1.4682 × 59,900) / (2.998 × 10^8)

t = approximately 587.20 microseconds.

For Event 4 at 59.909 km:

t = (2 × 1.4682 × 59,909) / (2.998 × 10^8)

t = approximately 587.29 microseconds.

3.5 Location Uncertainty

Given group-index uncertainty:

Delta n = ±0.002.

The distance estimated from OTDR time is inversely proportional to the group index. For a small group-index error:

Absolute distance error ≈ L × (Delta n / n).

At Event 1:

Distance error ≈ 12,000 × (0.002 / 1.4682)

Distance error ≈ ±16.3 m.

At Event 2:

Distance error ≈ 31,500 × (0.002 / 1.4682)

Distance error ≈ ±42.9 m.

At Event 3:

Distance error ≈ 59,900 × (0.002 / 1.4682)

Distance error ≈ ±81.6 m.

At Event 4:

Distance error ≈ 59,909 × (0.002 / 1.4682)

Distance error ≈ ±81.6 m.

These values describe the uncertainty due to group-index error alone. They do not include instrument timing error, fibre-length uncertainty, or event-detection uncertainty.

3.6 Connector Selection and Repair Verification

For the patch-panel repair, an LC connector is suitable where high-density data-centre connectivity is required. A CS connector may be considered for higher-density modern pluggable systems, while SC is a larger-format alternative. MPO is generally used for multi-fibre connections rather than as a direct replacement for an individual LC connector.

UPC connectors use a physical-contact polished end face and are commonly used in data communications. APC connectors use an angled end face and generally provide lower back reflection, but APC and UPC connectors must never be mated directly to each other. The repair must use compatible connector types, suitable cleaning procedures, and the correct mating interface.

After repair, repeat the OTDR test from both ends where possible, using suitable pulse widths. Verify the connector and splice losses with an optical power meter and inspect connector end faces before connection.

DESIGN DECISION 2 – OTDR AND REPAIR CONFIGURATION

I recommend prioritizing Event 3 because its 1.20 dB loss and -22 dB reflectance indicate a serious connector defect. Event 2 should then be repaired by replacing the high-loss mechanical splice with a fusion splice. Event 4 must be inspected and tested using short-pulse OTDR measurements because its 9 m separation from Event 3 makes its loss difficult to resolve. Repair acceptance must be based on post-repair OTDR and optical power-meter measurements.

- STAGE 4 – EYE DIAGRAM, BER AND QUANTUM-LIMIT ANALYSIS

4.1 BER from Q-Factor

The supplied relation is:

BER = 0.5 × erfc(Q / sqrt(2)).

For Channel 3:

Q = 7.6.

BER = 0.5 × erfc(7.6 / sqrt(2)).

BER is approximately 1.5 × 10^-14.

Therefore, Channel 3 satisfies the target BER of 10^-12 according to the supplied Q-factor relation.

For Channel 5:

Q = 4.7.

BER = 0.5 × erfc(4.7 / sqrt(2)).

BER is approximately 1.3 × 10^-6.

Therefore, Channel 5 fails the BER target of 10^-12.

The required Q-factor is 7.03. Channel 3 has Q = 7.6 and passes this criterion, while Channel 5 has Q = 4.7 and fails.

4.2 Total Jitter and Eye Width

The supplied relation is:

TJ = DJ + 14.07 × RJ.

For Channel 3:

Deterministic jitter, DJ = 0.18 UI peak-to-peak.

Random jitter, RJ = 0.012 UI RMS.

TJ = 0.18 + (14.07 × 0.012)

TJ = 0.18 + 0.16884

TJ = 0.34884 UI.

Approximate eye width = 1 − TJ

Eye width = 1 − 0.34884

Eye width = 0.65116 UI.

For Channel 5:

DJ = 0.30 UI peak-to-peak.

RJ = 0.022 UI RMS.

TJ = 0.30 + (14.07 × 0.022)

TJ = 0.30 + 0.30954

TJ = 0.60954 UI.

Approximate eye width = 1 − TJ

Eye width = 1 − 0.60954

Eye width = 0.39046 UI.

Channel 5 has greater total jitter and a smaller calculated horizontal eye opening. Channel 3 has an approximate eye width of 0.651 UI, while Channel 5 has an approximate eye width of 0.390 UI.

The eye-width calculation estimates the horizontal opening based on the supplied total-jitter model. It does not quantify vertical eye opening or guarantee BER independently.

4.3 Extinction Ratio and Channel-Specific Fault Diagnosis

Channel 3 has an extinction ratio of 9.0 dB, which exceeds the required 8.2 dB.

Channel 5 has an extinction ratio of 4.5 dB, which is 3.7 dB below the required value.

Extinction-ratio difference = 9.0 − 4.5 = 4.5 dB.

Channel 5 also has higher deterministic jitter, higher random jitter, lower Q-factor, and a more closed eye than Channel 3. Since both channels use the same fibre, these differences strongly indicate a channel-specific transmitter or transceiver fault rather than a purely shared fibre fault.

However, the shared fibre and connector defects can still reduce receiver margin for both channels. The appropriate procedure is to repair the common link defects and independently test Channel 5's transmitter, modulator bias, extinction ratio, driver, receiver alignment, and electrical timing.

4.4 Ideal Direct-Detection Quantum Limit

The supplied relation is:

BER = 0.5 × exp(-N1).

For a target BER of 10^-12:

10^-12 = 0.5 × exp(-N1).

Multiply both sides by 2:

2 × 10^-12 = exp(-N1).

Taking natural logarithms:

-N1 = ln(2 × 10^-12).

N1 = -ln(2 × 10^-12).

N1 = approximately 26.94 photons per bit.

This is the ideal photon number from the supplied direct-detection expression.

The energy per photon is:

E_photon = h × c / lambda.

Given:

h = 6.626 × 10^-34 J·s.

c = 2.998 × 10^8 m/s.

Assumed wavelength = 1550 nm = 1.55 × 10^-6 m.

E_photon = (6.626 × 10^-34 × 2.998 × 10^8) / (1.55 × 10^-6).

E_photon = approximately 1.282 × 10^-19 J.

Ideal received energy per bit:

E_bit = N1 × E_photon.

E_bit = 26.94 × 1.282 × 10^-19.

E_bit = approximately 3.454 × 10^-18 J/bit.

At 10 Gb/s, ideal optical power is:

P = E_bit × Bit rate.

P = 3.454 × 10^-18 × 10 × 10^9.

P = approximately 3.454 × 10^-8 W.

P = 34.54 nW.

Convert to dBm:

P_dBm = 10 log10(P in mW).

P_dBm = 10 log10(3.454 × 10^-5).

P_dBm = approximately -44.62 dBm.

The receiver sensitivity is -24 dBm.

Quantum-limit gap = -24 − (-44.62).

Quantum-limit gap = approximately 20.62 dB.

Thus, the specified receiver sensitivity is approximately 20.62 dB above the ideal direct-detection photon-counting limit under the supplied model. Actual receiver performance is limited by practical noise, finite extinction ratio, detector efficiency, bandwidth, implementation losses, and other non-ideal effects.

4.5 Homodyne and Heterodyne PSK Limits

The supplied homodyne relation is:

BER = 0.5 × erfc(sqrt(2N)).

For BER = 10^-12:

erfc(sqrt(2N)) = 2 × 10^-12.

Using the inverse complementary error function:

sqrt(2N) = erfc^-1(2 × 10^-12).

N = [erfc^-1(2 × 10^-12)]^2 / 2.

N is approximately 13.9 photons per bit.

The supplied heterodyne relation is:

BER = 0.5 × erfc(sqrt(N)).

For BER = 10^-12:

sqrt(N) = erfc^-1(2 × 10^-12).

N = [erfc^-1(2 × 10^-12)]^2.

N is approximately 27.8 photons per bit.

Using the same 1550 nm photon energy:

Homodyne ideal power ≈ 13.9 × 1.282 × 10^-19 × 10 × 10^9.

Homodyne ideal power ≈ 1.78 × 10^-8 W.

Homodyne ideal sensitivity ≈ -47.50 dBm.

Heterodyne ideal power ≈ 27.8 × 1.282 × 10^-19 × 10 × 10^9.

Heterodyne ideal power ≈ 3.56 × 10^-8 W.

Heterodyne ideal sensitivity ≈ -44.49 dBm.

These values are approximate theoretical comparisons based on the equations supplied in the case study. They are not practical receiver sensitivity specifications. Real coherent receivers have additional implementation penalties, and direct comparison requires consistent definitions of photons per bit, signal encoding, detection convention, and BER.

DESIGN DECISION 3 – EYE AND BER PERFORMANCE

I recommend replacing or recalibrating the Channel 5 transmitter or transceiver and verifying its extinction ratio, jitter, and eye opening after repair. Channel 5 currently fails the target because Q = 4.7 is below 7.03, and its estimated BER is approximately 1.3 × 10^-6. Channel 3 meets the Q-factor and BER requirements under the supplied relation but still needs adequate receiver margin. The ideal direct-detection quantum-limit calculation shows that practical sensitivity is substantially above the theoretical photon-counting limit.

- STAGE 5 – INTEGRATED WDM UPGRADE AND ENERGY STRATEGY

5.1 Required Upgrade Capacity

The target aggregate capacity is 1.6 Tb/s.

Option P:

Number of channels = 16.

Rate per channel = 100 Gb/s.

Aggregate capacity = 16 × 100 = 1600 Gb/s = 1.6 Tb/s.

Option Q:

Number of channels = 8.

Rate per channel = 200 Gb/s.

Aggregate capacity = 8 × 200 = 1600 Gb/s = 1.6 Tb/s.

Option R:

Number of channels = 4.

Rate per channel = 400 Gb/s.

Aggregate capacity = 4 × 400 = 1600 Gb/s = 1.6 Tb/s.

All three options meet the stated aggregate gross capacity.

5.2 ITU-T G.694.1 Frequency and Wavelength Calculation

The frequency-grid anchor is 193.1 THz.

For channel n = +3 on a 50 GHz grid:

Frequency = 193.1 THz + (3 × 0.05 THz).

Frequency = 193.25 THz.

The wavelength is calculated using:

Wavelength = c / Frequency.

Frequency = 193.25 × 10^12 Hz.

Wavelength = (2.998 × 10^8) / (193.25 × 10^12).

Wavelength = approximately 1.55175 × 10^-6 m.

Wavelength = approximately 1551.75 nm.

Therefore, the frequency is 193.25 THz and the wavelength is approximately 1551.75 nm.

5.3 Option Q Flex-Grid Slot Requirement

Option Q uses eight 200 Gb/s DP-16QAM channels with a nominal symbol rate of 32 GBd and a Nyquist roll-off factor of 0.1.

Approximate occupied bandwidth = Symbol rate × (1 + Roll-off factor).

Occupied bandwidth = 32 × (1 + 0.1).

Occupied bandwidth = 35.2 GHz.

Each flex-grid slot is 12.5 GHz.

Required slots per channel = Occupied bandwidth / Slot width.

Required slots per channel = 35.2 / 12.5.

Required slots per channel = 2.816.

The channel requires at least three 12.5 GHz slots, corresponding to a nominal allocated width of 37.5 GHz, assuming the stated slot allocation can accommodate the signal spectrum and implementation guard requirements.

Total allocated spectrum for eight channels = 8 × 37.5 GHz.

Total allocated spectrum = 300 GHz.

Payload spectral efficiency = Aggregate gross capacity / Allocated spectrum.

Payload spectral efficiency = 1600 Gb/s / 300 GHz.

Payload spectral efficiency = approximately 5.33 b/s/Hz.

This value is calculated using the rounded three-slot allocation per channel. It is not the same as the ideal modulation spectral efficiency before guard bands, framing, and FEC overhead.

5.4 Spectrum Comparison

Option P uses 16 channels on 50 GHz fixed slots.

Total nominal spectrum = 16 × 50 GHz = 800 GHz.

Gross spectral efficiency = 1600 Gb/s / 800 GHz.

Gross spectral efficiency = 2 b/s/Hz.

Option Q uses eight channels, each allocated three 12.5 GHz slots.

Total allocated spectrum = 8 × 37.5 GHz = 300 GHz.

Gross spectral efficiency = 1600 Gb/s / 300 GHz.

Gross spectral efficiency = approximately 5.33 b/s/Hz.

Option R uses four channels on fixed 75 GHz slots.

Total nominal spectrum = 4 × 75 GHz = 300 GHz.

Gross spectral efficiency = 1600 Gb/s / 300 GHz.

Gross spectral efficiency = approximately 5.33 b/s/Hz.

Option Q and Option R have the same nominal allocated spectrum under these assumptions, while Option Q has a lower stated OSNR requirement and higher total module power than Option R.

5.5 OSNR Margin Comparison

The given amplifier input power is -6 dBm per channel, noise figure is 5 dB, and supplied OSNR relation is:

OSNR = 58 + P_in − NF.

OSNR = 58 + (-6) − 5.

OSNR = 47 dB.

This is a calculated reference value from the supplied expression, not a verified end-to-end receiver OSNR. It must not be treated as the measured OSNR after the 60 km link.

Using 47 dB only as a provisional comparison:

Option P required OSNR = 12.5 dB.

Provisional OSNR margin = 47 − 12.5 = 34.5 dB.

Option Q required OSNR = 19.5 dB.

Provisional OSNR margin = 47 − 19.5 = 27.5 dB.

Option R required OSNR = 26 dB.

Provisional OSNR margin = 47 − 26 = 21 dB.

These margins are valid only if the 47 dB value represents the OSNR available at the relevant receiver reference plane and bandwidth. Actual receiver OSNR must be measured or calculated using the complete amplifier and fibre model before upgrade approval.

5.6 Module Power and Energy per Bit

Energy per bit is calculated using:

Energy per bit = Total module power / Aggregate bit rate.

For Option P:

Module count = 16.

Power per module = 22 W.

Total module power = 16 × 22 = 352 W.

Energy per bit = 352 W / (1.6 × 10^12 bit/s).

Energy per bit = 2.20 × 10^-10 J/bit.

Energy per bit = 220 pJ/bit.

For Option Q:

Module count = 8.

Power per module = 20 W.

Total module power = 8 × 20 = 160 W.

Energy per bit = 160 / (1.6 × 10^12).

Energy per bit = 1.00 × 10^-10 J/bit.

Energy per bit = 100 pJ/bit.

For Option R:

Module count = 4.

Power per module = 15 W.

Total module power = 4 × 15 = 60 W.

Energy per bit = 60 / (1.6 × 10^12).

Energy per bit = 3.75 × 10^-11 J/bit.

Energy per bit = 37.5 pJ/bit.

For the legacy 10G SFP+ reference:

Each module carries 10 Gb/s and consumes 3.5 W.

Energy per bit = 3.5 / (10 × 10^9).

Energy per bit = 3.5 × 10^-10 J/bit.

Energy per bit = 350 pJ/bit.

These calculations consider transceiver-module power only. They exclude amplifier pump power, switching, cooling, control electronics, and other network equipment.

5.7 Upgrade Comparison

Option P requires 16 modules and 800 GHz of nominal spectrum. Its stated OSNR requirement is 12.5 dB, and its calculated module energy is 220 pJ/bit.

Option Q requires eight modules and approximately 300 GHz of allocated spectrum using three 12.5 GHz slots per channel. Its stated OSNR requirement is 19.5 dB, and its calculated module energy is 100 pJ/bit.

Option R requires four modules and 300 GHz of nominal spectrum. Its stated OSNR requirement is 26 dB, and its calculated module energy is 37.5 pJ/bit.

Option R has the lowest module power and energy per bit, but it has the highest OSNR requirement. Option Q provides a useful balance between spectral efficiency, module count, energy consumption, and OSNR tolerance. The final selection must consider the measured receiver OSNR and the capabilities of the available transceivers.

DESIGN DECISION 4 – INTEGRATED WDM UPGRADE

I recommend Option Q as the balanced upgrade candidate because it provides 1.6 Tb/s using eight 200 Gb/s channels, approximately 300 GHz of allocated spectrum, and a calculated module energy of 100 pJ/bit. It has a lower OSNR requirement than Option R, while using substantially less spectrum than Option P. However, the operator must verify the actual receiver OSNR, transceiver interoperability, flex-grid support, and post-repair link margin before deployment. Option R should be considered if minimum module power is the dominant objective and its 26 dB OSNR requirement can be met with adequate margin.

- STAGE 6 – FINAL VERIFICATION TABLE

Parameter 1: Aggregate capacity.

Target = 1.6 Tb/s.

Evidence = Options P, Q, and R each provide 1.6 Tb/s based on the stated channel count and per-channel rate.

Parameter 2: Known as-built optical loss.

Calculated known loss = 16.52 dB.

Evidence = Fibre attenuation, measured connector losses, fusion-splice losses, mechanical-splice loss, and Event 3 loss. Event 4 loss remains unresolved.

Parameter 3: Receiver power.

Calculated received power = -21.52 dBm before additional penalties.

Receiver sensitivity = -24 dBm.

Calculated margin = 2.48 dB.

Required margin = 3 dB.

Evidence = The known-loss budget falls short by 0.52 dB before additional penalties. Repair and remeasurement are required.

Parameter 4: OTDR fault condition.

Event 2 loss = 0.62 dB.

Event 3 loss = 1.20 dB.

Event 4 loss = unresolved.

Evidence = Event 3 is the highest-priority known-loss event, Event 2 requires mechanical-splice replacement, and Event 4 requires improved local measurement.

Parameter 5: Channel 3 performance.

Q = 7.6.

Estimated BER = approximately 1.5 × 10^-14.

Extinction ratio = 9.0 dB.

Total jitter = 0.34884 UI.

Estimated eye width = 0.65116 UI.

Evidence = Channel 3 meets the supplied Q-factor, BER, and extinction-ratio requirements but still requires adequate receiver margin.

Parameter 6: Channel 5 performance.

Q = 4.7.

Estimated BER = approximately 1.3 × 10^-6.

Extinction ratio = 4.5 dB.

Total jitter = 0.60954 UI.

Estimated eye width = 0.39046 UI.

Evidence = Channel 5 fails the BER, Q-factor, and extinction-ratio targets and requires channel-specific corrective action.

Parameter 7: PMD.

Calculated PMD delay = approximately 0.775 ps.

Permitted value under the 10% bit-period criterion = 10 ps.

Evidence = The estimated PMD is within the stated tolerance.

Parameter 8: Quantum-limit comparison.

Ideal direct-detection sensitivity = approximately -44.62 dBm at 10 Gb/s and 1550 nm under the supplied model.

Specified receiver sensitivity = -24 dBm.

Calculated gap = approximately 20.62 dB.

Evidence = The practical sensitivity is above the theoretical limit, as expected for a non-ideal receiver.

Parameter 9: Upgrade spectrum.

Option P = 800 GHz.

Option Q = approximately 300 GHz.

Option R = 300 GHz.

Evidence = Option Q and Option R have the lowest nominal allocated spectrum under the given slot assumptions.

Parameter 10: Module energy per bit.

Option P = 220 pJ/bit.

Option Q = 100 pJ/bit.

Option R = 37.5 pJ/bit.

Legacy 10G SFP+ = 350 pJ/bit.

Evidence = Option R has the lowest stated module energy, while Option Q provides a balance between energy and OSNR requirements.

- FINAL DIAGNOSTIC AND UPGRADE WORKFLOW

Step 1: Measure the existing link using an optical power meter, OSA, OTDR, and eye/BER tester.

Step 2: Confirm the actual component inventory and reconcile the design connector count with the field measurements.

Step 3: Repair Event 3, inspect Event 4, and replace Event 2's high-loss mechanical splice with a fusion splice where appropriate.

Step 4: Repeat OTDR and power-meter measurements and verify that the repaired link meets the required loss and receiver margin.

Step 5: Diagnose Channel 5 independently by checking transmitter bias, extinction ratio, driver performance, receiver alignment, and jitter.

Step 6: Confirm Q-factor, BER, eye opening, and OSNR for all channels.

Step 7: Compare Options P, Q, and R against actual receiver OSNR, spectrum allocation, module power, and transceiver compatibility.

Step 8: Implement the selected upgrade, verify the channel plan, and repeat optical power, OSNR, BER, and performance measurements.

- FINAL ENGINEERING DECISION

DECISION: NO – NOT YET APPROVED FOR DEPLOYMENT.

The existing link should not be upgraded before its connector and splice defects are repaired and Channel 5 is restored to the required performance. The known as-built loss is 16.52 dB, the estimated received power is -21.52 dBm, and the available sensitivity margin is 2.48 dB, below the required 3 dB. Event 3 contributes the largest known localized loss at 1.20 dB, while Event 2 contributes 0.62 dB and should be replaced with a suitable fusion splice.

Channel 5 fails the BER target with an estimated BER of approximately 1.3 × 10^-6, compared with the required 10^-12. Its Q-factor of 4.7 and extinction ratio of 4.5 dB indicate a significant channel-specific problem. The transmitter or transceiver must be corrected and retested.

Option Q is the preferred balanced upgrade candidate because it provides 1.6 Tb/s using approximately 300 GHz of allocated spectrum and a calculated module energy of 100 pJ/bit. Option R has the lowest module energy at 37.5 pJ/bit but requires a higher OSNR of 26 dB. Final selection depends on verified receiver OSNR, available equipment, interoperability, and operational margin.

The link should be approved for deployment only after the repaired system achieves the required receiver margin, all channels meet the BER target, the selected upgrade satisfies its OSNR requirement, and all measurements confirm reliable 1.6 Tb/s operation.

REFERENCES

- ITU-T, Recommendation G.652, Characteristics of a Single-Mode Optical Fibre and Cable. Available at: https://www.itu.int/rec/T-REC-G.652/en

- ITU-T, Recommendation G.694.1, Spectral Grids for WDM Applications: DWDM Frequency Grid. Available at: https://www.itu.int/rec/T-REC-G.694.1/en

- IEC 61280-4-2, Fibre-Optic Communication Subsystem Test Procedures: Installed Single-Mode Fibre Cable Plant Attenuation and Optical Return Loss Measurement.

- IEC 61746, Calibration of Optical Time-Domain Reflectometers (OTDR).

- Agrawal, G. P., Fiber-Optic Communication Systems, 4th Edition, Wiley, 2010.

- Senior, J. M., and Jamro, M. Y., Optical Fiber Communications: Principles and Practice, 3rd Edition, Pearson, 2009.